Table 1: Summary of studies employing aerial remote sensing using visible band imagery only for large wood detection and quantification in rivers and river deltas. Many quantities have been estimated due to a lack of reporting of the dimensions of regions mapped, image pixel size, and other important details. See also Table 1 in Sendrowski and Wohl (2021), which lists other studies utilizing elevation-based and hyper-spectral imagery methods.

| Study | Data source | Site | Mapped active channel (ha) | Number of surveys (total time) | Wood detection methods | Mapped units | Pixel horizontal footprint (m) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Marcus et al. (2002) | Fixed wing plane aerial nadir imagery (estimated) | Yellowstone, USA | ~ 100 ha | Not clear. Several surveys on several rivers | Semi-automated (details lacking) | Large wood | Not reported, estimated < 1 m |
| Haschenburger and Rice (2004) | Fixed wing plane aerial nadir imagery | Carnation Creek, BC, Canada | ~63 ha | One (details lacking) | Manual | Large Wood | Not reported, estimated < 1 m |
| Smikrud and Prakash (2006) | Fixed wing plane aerial nadir imagery | Unuk River, AK, USA | 32 ha | Two (2003 – 2004) | Semi-automated supervised classification | Large Wood | 0.8 m |
| Lassettre et al. (2008) | Aerial imagery | Ain River, France | ~800 ha | Several (1945 to 2000) | Manual | Large Wood | Not reported, estimated < 1 m |
| Ortega-Terol et al. (2014) | Aerial imagery | Jucar River, Spain | ~ 2640 ha | One | Manual, semi-automated supervised classification | Large Wood | 0.25 m |
| Atha (2014) | Satellite | Queets River, WA, USA | ~ 730 ha | One (composite of public satellite imagery) | Manual | Large Wood | Not reported, estimated < 10 m |
| Ulloa et al. (2015) | Google Earth imagery, aerial imagery, and satellite image | Blanco River, Chile | ~ 102 ha | Three (2005, 2009, 2012.) | Manual and semi-automated supervised classification | Large Wood | 0.5 – 2.4 m |
| Taminga et al. (2015) | Orthoimagery from small UAV (quadcopter) | Elbow River, Alberta, Canada | 30 ha | One (Sept. 2012) | Manual | Large Wood | 0.05 m |
| Boivin et al. (2015) | Satellite and aerial imagery | St. Jean River, Quebec, Canada | ~ 500 ha | Numerous (1963 – 2013) | Manual | Large Wood | 0.63 – 15 m |
| Kramer et al. (2017) | Satellite and aerial imagery | Slave River, NWT, Canada | 4 sample locations in an estimated 6489 ha region | 59 (1983 – 2014) | Manual and semi-automated supervised classification | Large Wood | Not reported, estimated < 10 m |
| Sanhueza et al.(2019) | Orthoimagery from small UAV (quadcopter) | Blanco River, Chile | 1.2 ha | One (Mar-Apr 2016) | Manual | Large Wood | 0.016 m |
| Tsunetaka et al. (2021) | Orthoimagery from small UAV (quadcopter) | Otoishi River, Kyushu, Japan | 35.2 ha | Two (2018 – 2019) | Manual | Large Wood | 0.03 – 0.05 m |
| Sendrowski et al. (2023) | Satellites (GeoEye, Quickbird, and WorldView) | Mackenzie River Delta, AK, USA | 35,000 ha | Not reported (combined 2009 – 2021) | Manual delineation to train fully automated deep learning model. Manual post-processing. | Large Wood | 0.3 – 0.65 m |
| Iroumé et al. (2023) | Orthoimagery from small UAV (quadcopter) | Blanco Este River, Chile | 10.9 ha | 17 (2017-2020) | Manual | Large Wood | 0.02 m |
| This study | Orthoimagery from fixed wing airplane | Elwha River, WA, USA | 246 ha | 14 (2012 – 2017) | Manual and semi-automated delineation to train fully automated deep learning model | Large Wood, Sediment, Water, Vegetation | 0.125 m |